Department of Computer Science

FAST – National University of Computer & Emerging Sciences Chiniot-Faisalabad

Computer Networks – Semester Project Fall 2025

NU-Information Exchange System (FAST-NUCES Multi-Campus Network)

Team

Nimrah Shahid                     23F-0734

Aqsa Ishaq                           23F-0839

Ayesha Rauf                         23F-0807

Date of Submission: 12/8/2025

Submitted to

Mam Rabia Anwer

Table of Contents

1. Introduction	3

2. Campus Overview	3

3. Addressing Plan	3

4. VLAN & Departmental Structure	4

5. Wireless Network Design	4

6. Routing Design	5

7. WAN Design	5

8. DHCP Design	6

9. Security Measures	6

10. Complete Text Topology Diagram	7

11. CLI Configuration	10

12. Why This Design is the Best	12

## 1. Introduction

This report presents a complete network design for a multi-campus university consisting of six campuses, interconnected through a central core network, providing secure, scalable, and high-performance connectivity. The design uses:

- Private IP Block: 10.0.0.0/8

- Interior Routing Protocol: OSPF Multi-Area

- Exterior Routing Protocol (between campuses): EIGRP

- Layer-2 Domain: VLAN-based segmentation

- Wireless Management: Centralized WLC + Lightweight Access Points

- High Availability: Redundant links, dual routers, loop-free Layer-2

The objective is to create a centralized yet scalable network that supports thousands of students, staff, servers, and wireless users across all campuses.

## 2. Campus Overview

| Campus | Population | Subnet Assigned | OSPF Area | AP Count | Notes |
| --- | --- | --- | --- | --- | --- |
| Campus A | 1000 | 10.1.0.0/16 | Area 1 | 50 | Main campus, WLC hosted here |
| Campus B | 600 | 10.2.0.0/16 | Area 2 | 30 | Faculty/Library |
| Campus C | 400 | 10.3.0.0/16 | Area 3 | 20 | CS & Engineering |
| Campus D | 300 | 10.4.0.0/16 | Area 4 | 20 | Admin Campus |
| Campus E | 200 | 10.5.0.0/16 | Area 5 | 15 | Research Dept |
| Campus F | 150 | 10.6.0.0/16 | Area 6 | 10 | Remote campus |

## 3. Addressing Plan

3.1 Primary IP Block

10.0.0.0/8

3.2 Allocation per Campus

Each campus receives a /16, allowing up to 65,536 addresses.

| Campus | Subnet | Range |
| --- | --- | --- |
| A | 10.1.0.0/16 | 10.1.0.1 – 10.1.255.254 |
| B | 10.2.0.0/16 | 10.2.0.1 – 10.2.255.254 |
| C | 10.3.0.0/16 | 10.3.0.1 – 10.3.255.254 |
| D | 10.4.0.0/16 | 10.4.0.1 – 10.4.255.254 |
| E | 10.5.0.0/16 | 10.5.0.1 – 10.5.255.254 |
| F | 10.6.0.0/16 | 10.6.0.1 – 10.6.255.254 |

## 4. VLAN & Departmental Structure

Common VLANs Across All Campuses

| VLAN ID | Name | Purpose |
| --- | --- | --- |
| 10 | Admin | Staff offices |
| 20 | Faculty | Teachers |
| 30 | Student | Labs, classrooms |
| 40 | Servers | Datacenter machines |
| 50 | Wireless | AP management |
| 60 | Voice | IP Phones |
| 99 | Management | Switch/WLC management |

## 5. Wireless Network Design

WLC Deployment

- WLC placed in Campus A VLAN 50

- Manages all APs across six campuses

- AP mode: Lightweight (CAPWAP)

SSID Design

| SSID | VLAN | Security |
| --- | --- | --- |
| Uni-Student | 30 | WPA2-Enterprise |
| Uni-Faculty | 20 | WPA2-Enterprise |
| Uni-Guest | 50 | WPA2-PSK (separate DHCP pool) |

## 6. Routing Design

6.1 Interior Routing (Inside Each Campus): OSPF Multi-Area

OSPF Areas:

| Campus | Area ID |
| --- | --- |
| A | 1 |
| B | 2 |
| C | 3 |
| D | 4 |
| E | 5 |
| F | 6 |

- Each campus runs OSPF internally.

- Area 0 runs on the Core Router at Campus A.

- All campus routers connect to Area 0 using point-to-point WAN links.

6.2 Exterior Routing (Between Campuses): EIGRP

- Autonomous System Number: AS 100

- Used only between campus routers to exchange inter-campus routes.

- Redistribution between OSPF and EIGRP occurs at Campus A Core Router.

## 7. WAN Design

Campus Interconnections:

Campus A ↔ B

Campus A ↔ C

Campus A ↔ D

Campus A ↔ E

Campus A ↔ F

- Backbone routers use /30 links:

  - 10.255.x.x/30

Example:

A–B Link → 10.255.1.0/30

A–C Link → 10.255.2.0/30

## 8. DHCP Design

DHCP Servers exist in Campus A (Centralized)

- Router-on-a-Stick subinterfaces forward DHCP requests via ip helper-address.

- DHCP Pools for:

  - Admin

  - Student

  - Faculty

  - Servers

  - Wireless

  - Guest WiFi

## 9. Security Measures

| Layer | Security Features |
| --- | --- |
| Layer 2 | Port security, BPDU Guard, Disabled DTP |
| Layer 3 | ACLs, OSPF Authentication, EIGRP Authentication |
| Perimeter | NAT overload for Internet |
| Wireless | WPA2 Enterprise, Guest VLAN isolation |
| Management | SSH only, SNMPv3 |

## 10. Complete Text Topology Diagram

## 11. CLI Configuration

## 12. Why This Design is the Best

✔ Scalable

Using 10.0.0.0/8 with /16 per campus supports future population growth.

✔ Fast Convergence

OSPF Multi-Area ensures efficient routing inside each campus.

✔ Robust Inter-Campus Connectivity

EIGRP gives high performance and load balancing across campuses.

✔ Centralized Wireless Control

WLC in Campus A reduces management overhead.

✔ High Security

ACLs + WPA2 Enterprise + OSPF/EIGRP authentication + VLAN segmentation.

✔ Easy Management

Campus A hosts all important services (DHCP, WLC, Core Routing).